[image: image1.jpg]STEEL DOORS




Flameguard Specification L20

PRIMA SINGLE / DOUBLE FLAMEGUARD DOOR

60 /120 / 240 FR to BS 476 Part 22

LEAF

        50mm thick Door leaf with flush lipped astragal constructed from:-

· 2 No. 1.2 mm thick zinc coated steel sheets, which are welded together

· Reinforcing perimeter channels

· 47.6 mm thick honeycomb core

FRAME 
-       Manufactured from 1.6 mm thick zinc coated steel
· Single rebate 130mm wide frame with additional rebate for flush lip

· A minimum of four fixing points are provided to each jamb

· Knockdown frames on double doors for transit and distribution

· 100, 200mm , 250mm are available upon request

DOOR IRONMONGERY

· BS 7352 Grade 13 stainless steel, twin bearing template drilled butt hinges with integral security dog bolts are fitted as standard with CE marking

· Geze TS4000 Door closer to meet Part B Building Regulations.

· Refer schedule for balance of ironmongery

VISION PANELS (Optional)

Single and Double glazed in Panel Design B, C, D and E 

6mm GWPP/ 6mm Clear pyrocet upto 1 hour

8.8mm Firelite Laminated 2-4 hours

24mm Double Glazed units available

LOUVRES (Optional)

-     40% Ventilation in Panel Design F, G, and H – Fire Block louvre system

FINISH

· Comes as standard in a powder applied primer finish.

· Powder coating is available as an option 

· Marine Grade finish is available as an option in salt laden coastal or high chlorination locations.

· Food Safe finish is available as an option

THRESHOLD (Optional)
· Deluxe aluminium threshold with neoprene seal on external outward opening doors

-     MWK / Ramped threshold for External inward opening doors with Expelex deflector


SEALS (Optional)
· Prima standard frame seals available upon request.

SIDEPANELS & OVERPANELS (Optional)
· Solid, Glazed and Louvred  side and overpanels available in a fixed or removable arrangement.    

MANUFACTURER

PRIMA DOORS LTD

NEWBY ROAD IND ESTATE

HAZEL GROVE

STOCKPORT

CHESHIRE

SK7 5DA

EMAIL: info@primadoors.co.uk 

WEB :   www.primadoors.co.uk

