[image: image1.jpg]STEEL DOORS




Sportsguard Specification L20

PRIMA SINGLE / DOUBLE SPORTSGUARD DOOR

LEAF

        50mm thick Door leaf with flush lipped astragal constructed from:-

· 2 No. 1.2 mm thick zinc coated steel sheets, which are welded together

· Reinforcing perimeter channels

· 47.6 mm thick honeycomb core
· Steel Rebound boards with infil  top and bottom to give flush finish with frame and inside block wall and conceal panic furniture
FRAME 

· Single rebate 120mm wide frame flush with inside blockwork 

· Cavity closers to outside of frame to cover cavity wall is available as an option
DOOR IRONMONGERY

· BS 7352 Grade 13 stainless steel, twin bearing template drilled butt hinges with integral security dog bolts are fitted as standard with CE marking
· Exidor 400 series Touchbar Pullman panic furniture recessed into rebound boards 

· Refer Schedule for balance of ironmongery

FINISH

· Comes as standard in a powder applied primer finish.

· Powder coating is available as an option 

· Marine Grade finish is available as an option in salt laden coastal or high chlorination locations.
· Food Safe finish is available as an option

THRESHOLD (Optional)
· Deluxe aluminium threshold with neoprene seal on external outward opening doors

SEALS (Optional)
· Prima standard frame seals available upon request.

MANUFACTURER
PRIMA DOORS LTD

NEWBY ROAD IND ESTATE

HAZEL GROVE

STOCKPORT

CHESHIRE

SK7 5DA

EMAIL: info@primadoors.co.uk 

WEB :   www.primadoors.co.uk

Tel No.
0161 487 3286

Fax No.
0161 487 3285

